- Why is the concept of Eccentric Loading important?

  - In most real world situations you will not find a single stress state

    - One of the most common situations found is an eccentric axial

    - Important to recognize all the types of stress present

    - Important to be able to know how to combine the different stresses to ensure a safe design

- Example

  - Show them cases of the combined loading situation on loader beam pictured on Combined Loading lesson page

- Lab Procedure

  - We will be applying an offset load to an aluminum beam to create a combined load

    - Each group only needs to test one of the available beams

  - Begin by measuring all the dimensions required by the data sheet

  - Draw on board

  - Do not assume gage #3 is centered on the beam

    - Make sure you measure the location of all gages

  - Use a square and level to align the beam

    - You will be shown how to align the beam in the lab

  - Apply a pre-load of 100-200 lb to hold the beam in place

  - Connect all 5 strain gages to the switch and balance unit

    - Zero the amp and set the gage factor on the indicator

    - Balance the load for each strain gage on the switch and balance unit

  - Zero the load reading

  - Apply an additional 8,000-10,000 lb and then read and record the strains on your data sheet

    - You will only take one set of readings from the strain gages

- Calculations

- Theoretical Calculations

  - Draw FBD of top part of beam on board

  - Cut the beam horizontally at the location of the strain gages

    - Need to find equivalent load condition at the centroid of our cut

  - Offset load creates both axial and bending stresses in the beam when we transfer the applied load to the cross section of interest

    - Apply superposition to combine the axial and bending stress distributions

    - We have effectively found the distribution of strain across the cross section with the strain gages

  - Find the theoretical stress vs. position line

    - Axial Stress

    - Bending Stress

        - Moment of inertia is found about the centroidal axis at the cross section of interest

    - Combined Stress

  - Calculate the theoretical neutral axis location by setting

  - Also calculate the max. tensile and compressive stresses using your equation for

    - Max. tensile 

  - Max. compressive 

- Experimental Calculations

  - Start calculations by inputting your data to Excel

    - Calculate the experimental stresses present in the beam

      - Uniaxial Hooke’s Law

    - Create a plot of stress vs. distance from the centroid

      - Refer to data sheet for the positive and negative directions for your distance measurements

      - Plot your theoretical stress vs. position equation on the same graph with your experimental regression line

Theoretical

    - Use linear regression to find the equation of the regression line for your experimental data

      - Result will be in the form

    - Find the location of the experimental neutral axis

      - Set  in your regression equation from the definition of neutral axis as the point with 0 stress

    - Calculate the maximum tensile and compressive stresses in the beam using your regression equation

      - Max. tensile stress is on the outside of the cross section at

      - Max. compressive stress is on outside of cross section at

- Lab Report

  - The report for this lab should be a memo written by your group worth 100 points

  - Attach your initialed data sheet and a set of hand calculations

  - Experimental Results

    - Include the graph of your data

      - Show both the linear regression line for your data and the theoretical stress vs. position line

    - Show the calculated values for the following

      - Experimental regression line equation

      - Theoretical stress vs. position equation

      - Experimental and theoretical neutral axis location

      - Experimental and theoretical max. tensile and compressive stresses

  - Discussion of Results

    - Compare your experimental values to the theoretical ones using percent difference

      - Should have 3 percent differences total

    - Discuss how well the theory matched  your experimental results

    - Give reasons for any major differences in your results

- Presentation

  - Each group will come to the board and fill in your experimental values for the following: neutral axis location, max. tensile stress, and max. compressive stress.

  - Then two random groups will be asked questions about the lab