Pressure Vessels

Lab Background

This week we will use strain gages to measure the strains on two different pressure vessels.  The measured strains will then be used to calculate the stresses present in the pressure vessel walls.  It is important to be able to measure the stresses in pressure vessel walls to ensure they are properly designed to prevent bursting or implosion.

Lab Procedure

Each group will be working with two different pressure vessels this week.  The first vessel is a thin-walled steel pressure vessel.  The second pressure vessel is thick-walled and made from aluminum.  The following is a brief outline of the procedure you should follow for each vessel:

Thin-Walled Steel Pressure Vessel

- Begin by observing and recording the orientation of the strain gage rosette on the vessel.  Be sure to label which gage is which on your data sheet.

- Measure the orientation angle of the strain rosette.  This angle should be measured as the angle between the axial direction of the cylinder and the gage closest to the axial direction (gage A).  Define the x’ axis to be along the same direction as gage A and the y’ axis to be along the direction of gage C.

- Zero the amp, set the gage factor, and balance the strains on the strain indicators.

- Close the pressure relief valve.

- Use the pump to increase the pressure inside the cylinder in 250 psi increments.  Take readings from all three strain gages at every 250 psi increment until the pressure reaches 2000 psi.

- When finished, open the pressure relief valve.

Thick-Walled Aluminum Pressure Vessel

- The two strain gages used for this vessel are oriented along two of the principal directions.  Gage #1 is in the hoop direction and gage #2 is in the radial direction.

- Zero the amp, set the gage factor, and balance the strains on the strain indicator.  You will only be able to balance the strain for one of the strain gages.  For the other gage, record the strain at 0 psi and subtract this value from all your other strain values.

- Close the pressure relief valve.

- Use the pump to increase the pressure in 125 psi increments.  Take readings from the two strain gages at each 125 psi increment.  Continue taking readings until the pressure reaches 1000 psi.

- When finished, open the pressure relief valve.

Calculations

The calculations for this lab are fairly involved, but by no means impossible to complete.  If you have any difficulties, feel free to contact me and I will help you with them.  If you follow the steps listed below, you should be able to complete your calculations without too much trouble.

- Use Excel to plot normal strain (y-axis) vs. pressure (x-axis) for each vessel.

- Use linear regression to find the slope of each normal strain vs. pressure data set on your graph.  There should be one line for each strain gage used.  This will give you values for,, and for the thin-walled vessel and values for (hoop direction) and (radial direction) for the thick walled vessel.

- Find the normal strains along the x’ and y’ axes and the shearing strain for the thin-walled vessel by applying the equations for a 0, 45, 90º strain rosette which are given in Eq. (1).

- For the thin-walled vessel, apply the biaxial form of Hooke’s Law given as Eq. (2) to calculateσx'p,σy'p, and τx'y'p.  The principal stresses in the thick-walled vessel can be found using a slightly different version of Hooke’s law shown as Eq. (3).

- For the thin-walled vessel, use either Mohr’s circle or the stress transformation equations to calculate the principal normal stresses per unit pressure.  will be the principal stress in the hoop direction and will be the principal stress in the axial direction.  You will already have the principal normal stresses per unit pressure for the thick-walled vessel from step 4.

- From either Mohr’s circle or the equations, calculate the orientation of the principal axes for the thin-walled vessel.

- Apply the thin-wall theory given in Eq. (4) to both of the vessels tested in the lab to find reference values for your results.

- Also apply the thick-wall theory given in Eq. (5) to both the thin-walled and thick-walled vessels to find a second set of reference values for your results.

Lab Report

The report for this lab should be a memo written by your group worth 100 points.  Be sure to attach your initialed data sheet.  A set of sample hand calculations should also be included.  The following describes what is expected in two sections of your report.

Experimental Results

Describe the method used in performing your calculations.  Include the two graphs created in Excel.  Make sure to show all the regression lines and their equations on the graph.  Also, describe how you found the reference values using the two pressure vessel theories.  Include a table or tables showing the following experimental values for the thin-walled vessel:

| εx'p | εy'p | γx'y'p | σx'p | σy'p | τx'y'p | σ1p | σ2p | θp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

For the thick-walled vessel create a table showing the following experimental values:

| ε1p | ε3p | σ1p | σ3p |
| --- | --- | --- | --- |

Discussion of Results

Compare your experimental principal stresses per unit pressure to the values predicted by the thin-walled and thick-walled theories.  You need to compare each experimental vessel to both of the theories.  Use percent errors to show how accurate the theories are.  Also, compare your calculated orientation of the principal axes to the actual orientation of the principal axes.  In theory, the principal axes should be directly aligned with the axial and hoop directions.  Tables should be used to help organize all your comparisons.

Presentation

Each group will write their experimental values for the following on the board:

| Vessel Type |  |  |  |
| --- | --- | --- | --- |
| Thin-Walled |  |  | N/A |
| Thick-Walled |  | N/A |  |

Then two random groups will be asked questions about the lab.

Equations

(1) 0, 45, 90º Strain Rosette	(2) Biaxial Hooke’s Law		(3) Biaxial Hooke’s Law
εAp=εx'p	σx'p=E1−ν2εx'p+νεy'p		σ1p=E1−ν2ε1p+νε3p

εCp=εy'p	σy'p=E1−ν2εy'p+νεx'p		σ3p=E1−ν2ε3p+νε1p

τx'y'p=Gγx'y'p			= principal hoop stress, psi

= normal strain, in/in	= normal strain, in/in		= principal radial stress, psi

= shear strain, rad	= shear strain, rad		= hoop strain, in/in (gage #1)

p= gage pressure, psi	p= gage pressure, psi		= radial strain, in/in (gage #2)

= normal stress, psi

= shear stress, psi

E= modulus of elasticity, psi

G= modulus of rigidity, psi

= Poisson’s ratio

(4) Thin-Wall Theory	(5) Thick-Wall Theory		(6) Elastic Constants Relationship

σ1p=at (hoop)	σ1p=a21+b2r2b2−a2(hoop)		G=E21+υ

σ2p=a2t(axial)	σ2p=a2b2−a2(axial)

σ3p=−1 (radial)			σ3p=a21−b2r2b2−a2(radial)

a = inner radius, in	a = inner radius, in

t = vessel wall thickness, in	b= outer radius, in

r = radius to gage, in