- Pressure Vessels

  - Any container that holds a fluid under a positive or negative internal pressure

    - Pressure may be well above or below atmospheric pressure

    - Vessels holding fluids under static pressure are also pressure vessels

  - Many are used in everyday life

LessonExamples

- Lab Procedure

  - You will perform tests on two different pressure vessels

    - Thin-walled and thick-walled

  - Thin-Walled Vessel Test

    - Pressure vessel is considered thin-walled if

    - We will use a 3 strain gage rosette to measure strain on the outside of the vessel wall as pressure is applied inside

      - Note the orientation of the 3 strain gages in the rosette

      - Should be 0, 45, and 90º

      - Also make sure you properly label which gage is A, B, and C for the data you collect

      - Draw on board

      - Orient the x’ axis along gage A and y’ axes along gage C

    - Measure the orientation angle using a protractor

      - Should be measured as the angle between the axial direction of the cylinder and the gage oriented the closest to the axial direction (gage A)

    - Zero the amp

    - Set the gage factor

    - Balance the strain

    - Close the pressure relief valve

    - Use the pump to increase the pressure in 250 psi increments from 0 psi to 2000 psi

      - Take readings from the 3 strain gages at each 250 psi increment

    - Once you reach 2000 psi and are finished taking readings, open the pressure relief valve

  - Thick-Walled Pressure Vessel Test

    - The two strain gages we will use are designated as #1 and #2 on your data sheet

      - These gages are aligned in two of the principal directions

        - #1 is aligned in the hoop direction

        - #2 is in the radial direction

    - Zero the amp, set the gage factor, and balance the strain for gage #1

    - Use the switch to change to gage #2

      - Do not balance the strain again

      - Write down what the strain is at 0 psi and subtract this value from all your others for gage #2

    - Close the pressure relief valve

    - Use the pump to increase the pressure in 125 psi increments from 0 psi to 1000 psi

      - Take strain reading for the two gages at each 125 psi increment

    - Open the pressure relief valve when finished

- Calculations

  - Thin-Wall Experiment

    - Begin by entering your data in Excel

    - Create a plot of normal strain vs. pressure with a line for each of your strain gages

      - Put all the lines on the same graph

    - Use linear regression to calculate the slope εip of the lines for each of your strain gages

      - Calculate the strains along the x’ and y’ axes along with the shearing strain

        - εAp=εx'p

        - εCp=εy'p

      - Above equations come from the fact that if we have 3 strain gages measuring strain at a given point

        - With each gage arbitrarily oriented we can use the following:

    - Gages are on surface of pressure vessel

      - In a state of plane stress

    - Use biaxial Hooke’s law to convert your strains into stresses

      - σx'p=E1−ν2εx'p+νεy'p

      - σy'p=E1−ν2εy'p+νεx'p

      - τx'y'p=Gγx'y'p

    - Relationship between elastic constants

      - G=E21+υ

    - Use Mohr’s circle or the equations method to find:

      - Principal normal stresses per unit pressure

        - σ1pwill be the principal stress in the hoop direction

        - σ2pwill be the principal stress in the axial direction

      - Orientation angle of the principal axes with respect to the x’ axis

        - θp should be the same as

  - Thick-Wall Experiment

    - Much simpler than the thin-walled vessel

    - Gage #1 directly measures principal strain in the hoop direction ε1

    - Gage #2 directly measures the principal strain in the radial directionε3

    - Again, create a strain vs. pressure plot and find the slope of the two lines on your graph

    - Use Hooke’s law to calculate principal stresses using the measured principal strains

      - σ1p=E1−ν2ε1p+νε3p

      - σ3p=E1−ν2ε3p+νε1p

    - Do not worry about the maximum shear stresses for the thick wall vessel

  - Theoretical Equations- Reference Values

    - Thin-Wall Pressure Vessel

      - Equations are applicable if at≥10

      - The thin-walled equations neglect the radial stresses in the wall by assuming none are present due to the thin wall

      - We will use them as a reference for both vessels to show that they fail miserably for a thick-walled vessel

        - σ1p=at (hoop)

        - σ2p=a2t(axial)

        - σ3p=−1 (radial)

    - Thick-Wall Pressure Vessel

      - Equations take into account radial stresses

        - σ1p=a21+b2r2b2−a2(hoop)

        - σ2p=a2b2−a2(axial)

        - σ3p=a21−b2r2b2−a2 (radial)

      - These equations work for both thin and thick-walled vessels

        - Note that for the thin-walled vessel r = b

- Lab Report

  - Memo completed by your group worth 100 points

    - Attach your initialed data sheet

    - Also attach a set of hand calculations

  - Experimental Results

    - Thin-walled vessel

      - Show the graph you will create from your data

      - Include a table or tables showing your calculated experimental values for the following:

| εx'p | εy'p | γx'y'p | σx'p | σy'p | τx'y'p | σ1p | σ2p | θp |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

      - Also include a table showing the reference values of σ1pand σ2pfor the thin-walled vessel using both the thin-wall and thick-wall theory

        - Use a % error to compare your experimental values to the reference values

    - Thick-Walled Vessel

      - Show the graph you will create from your data

      - Include a table showing the following calculated experimental values:

| ε1p | ε3p | σ1p | σ3p |
| --- | --- | --- | --- |

      - Also include a table showing the reference values for σ1pand σ3pfound using both the thin-wall and thick-wall theories

        - Use a % difference to compare the theoretical values to your experimental values

  - Discussion of Results

    - Compare your experimental principal stresses to those found using the two theories

      - You need to compare your experimental results from each vessel to both of the theories

      - Use a percent error

      - Discuss how well the theories work

        - In particular mention if the thin-walled theory is appropriate for use on thick-walled vessels

    - Compare the calculated principal direction for the thin-walled vessel to the measured orientation angle

      - In theory these should be the same

- Presentation

  - Each group will come to the board and fill in their experimental values for the following:

| Vessel Type |  |  |  |
| --- | --- | --- | --- |
| Thin-Walled |  |  | N/A |
| Thick-Walled |  | N/A |  |

    - Then two random groups will be asked questions.