AutoCAD Lesson #1:

#### The Basics of Two Dimensional AutoCAD

Capture the screens and show students where to find the AutoCAD templates.  Show the students where AutoCAD is on the desktop and how to access it from the start/programs/ menu (2 minutes).

Start up AutoCAD and open the 2D template.  (2 minutes).

Unlock the screens and let the students log-in, start AutoCAD, open the template.  Walk around and assist the students with problems (4 minutes).

Recapture the screens and do a demonstration of AutoCAD (25 min).

# Show the following, and tell students the basics about each.

Model vs. Paper Space:

- Model Tab (Black Background):  A possible analogy for this drawing option is ‘an infinite plane for two-dimensional drawings, such that you can draw anywhere, and then you can zoom in or out with your ‘picture frame’ to find it.  The chalkboard would be an example of an infinite plane where any shape could be drawn at any location, but you may not be able to see that shape based on where your ‘picture frame’ is located.

- Layout1 Tab (next to Model Tab):  The 2D template provides the drawing border in this option, with a white background.  This border could be analogous to the picture frame mentioned above.

- Model Space (on Status Bar, Layout 1 tab, model border):  Should do all drawing in model space, white background, drawing border the border is the picture frame’ that can be moved anywhere and resized to fit your drawing

- Paper Space (on Status Bar, Layout 1 tab, paper border):  ‘Freezes’ the image inside the border.  Text Items such as your name, title, etc, can be used here with a ddedit command.  This is the mode that you will print in.

Toolbars:  Show some of the basic toolbars and how to view hidden toolbars by right clicking while in a visible toolbar to get a list of all toolbars which can then be turned on.

Status Bar:  Show the following:

- Object Snap – Causes the cursor to grab onto geometric figures such as endpoints and circle centers.  Show different modes on OSNAP (right click on snap then select preferences).

- Polar Tracking – Displays an alignment path whenever the direction angle is in an increment of 90 degrees {the 90 degrees setting can be changed}.

- Grid – Makes a grid in Model Space.  The x and y increments can be changed to make scaling a drawing easier.

- Snap –  Allows the cursor to move only in certain increments (for ease of drawing).  Show students the difference in how the cursor moves with and without snap.

Command Line:  Show how commands may be entered on the command line,

explain that it will give instructions during many operations, and that it has a history of what commands have been performed by scrolling upwards (show).

Escape Key:   Cancels active commands.  If something doesn’t work like you

think it should, check the command line and hit the escape key.

Layers:   Show how to switch between layers on the 2D template.  Using the template, a student should not need to change any layer properties.  If a student should draw something in the wrong layer, the layer of the line should be changed, not the properties of the line itself.

## Draw the following and explain each step.

Line Segments:  Various ways of making the lines:

- Cartesian Coordinates x-value, y-value

- Relative Coordinates @x-value,y-value (from original point)

- Polar Coordinates @distance<angle

- Talk about frustration of making a straight line freehand; SNAP reminder

Circles:   Six methods of drawing circles.  The most common are:

- Center and Radius (used most often) – specify center, then radius

- Center and Diameter

- 2 and 3 point circles

- Center marks can be created by entering d on the command line.  From the dialogue box, select MODIFY, and another dialog box should appear.  Select LINE for your drawings.

Arcs (Chords):   Eleven methods.  Go over the most common:

- 3 point Arcs (starting point, any other point in arc, endpoint) ~ default type

- Start, Center, Included Angle – ccw angle is positive

- Start, Center, End – Constructs an arc counterclockwise

Rectangles:  Construct a rectangle by specifying two opposite points.

Can use same types of drawing points as the line segment

##### Drawing Tools

Chamfer:   Show its location on the side toolbar and how to enter it as a

Command.  Show how to specify and check the dimensions of the cut for both lines.  Show how to select the lines in the right order for chamfering.

Fillet:   Show its location on the toolbar, show how to select the appropriate lines

to fillet and if you specify the radius of the circle you must enter the command again to execute it.

Erase:   Show how to use the erase command and how to select the objects to be

erased.

Undo:  Undoes the last items without leaving the command.  Redo works one

Time to redo the undo !

Oops:  A command that you enter so that you can undo the last erase.

Show examples of how this can be very helpful.

## Plotting (Printing) and Saving

Printing:

- Remind Students how to change back to Paper Space to print their drawing.

- Select the appropriate plotting device (specify which one it is).

- Tell students they will be required to know how to plot to the CLC printer for exams.  The print format is to print using a window to determine the plot area

- Printing on ‘Scaled to fit’ or a specified zoom.

### Saving

Remind Students how to save to their S:/ drive often (in case of a computer crash).  Give hints on how to name their drawings.

Release the screens and have the students practice drawing various shapes, chamfering, filleting, erasing, printing, and saving.  Walk around the room to assist students with problems.