CE 120 – HARDNESS TESTING

| Section (circle): | A B C D E F G H I J K | Date: |  |  |
| --- | --- | --- | --- | --- |
| Group (circle): | 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 |  |  |  |
| Data Recorder: |  |  |  |  |

Metal Specimen, Rockwell Test

| Procedure: (circle one) Rockwell B: 1/16" indenter, 100 kg major load, red dial Rockwell C: diamond indenter, 150 kg major load, black dial |  |
| --- | --- |
| Material (see table on next page): |  |
| Size: |  |
| Anvil (flat for rectangular or vee for cylindrical): |  |
| Reference Rockwell B or C value: |  |
| Reference tensile strength: | ksi |
| Three experimental readings: | / / |
| Average of three experimental readings: |  |
| Corrected experimental value (cylindrical specimens only), from wall chart or ASTM E18 Table 11 or 12: |  |
| Tensile strength, from wall chart or ASTM E140: | ksi |

Metal Specimen, Brinell Test

| Material (see table on next page): |  |
| --- | --- |
| Reference Brinell hardness value: |  |
| Reference tensile strength: | ksi |
| Load, P: | 3000 kg |
| Indenter diameter, D: | 10 mm |
| Indentation diameter, d: | mm |
| Brinell hardness from table on testing machine or formula: |  |
| Tensile strength, from wall chart or ASTM E140: | ksi |

Polymer Specimen, Durometer or Rockwell Test

| Procedure: (circle one) Durometer A2: analog, operating stand Durometer A: analog, hand-held Durometer D: analog, hand-held Durometer M: electronic, operating stand; Durometer Foam: analog, hand-held Rockwell M: 1/4" indenter, 100 kg major load, red dial Rockwell R: 1/2" indenter, 60 kg major load, red dial. |
| --- |
| Material (see table below): |  |  |
| Reference hardness value: |  |  |
| Three experimental readings: | / / |  |
| Average of three experimental readings: |  |  |

Materials

| Material | Reference Hardness | Reference Tensile Strength (ksi) | Available Sizes | Identification Color or Code |
| --- | --- | --- | --- | --- |
| C11000-H2 Copper | Rockwell B: 40 | 41.2 | 5/8"x6" | blue |
| 6061-T651 Aluminum | Brinell: 95 Rockwell B: 60 | 45 | 1/2"x3"x3" 5/8"x6" | blue-green, AL 6061 |
| C36000-H1 Brass | Rockwell B: 62 | 55.8 | 1/2"x3"x3"  5/8"x6" | orange, BRS 36000 |
| A36 Hot-Rolled Steel | Brinell: 119-159 Rockwell B: 67-83 | 71 (58-80 is common) | 1/2"x3"x3"  5/8"x6" | green-white, ST A36 |
| C1018 Cold-Drawn Steel | Brinell: 126 Rockwell B: 71 | 63.8 | 1/2"x3"x3" | red-white, ST 1018 |
| 2024-T4 Aluminum | Brinell: 120 Rockwell B: 75 | 68.2 | 1/4"x1"x6"  5/8"x6" | blue-red, AL 2024 |
| 7075-T651 Aluminum | Brinell: 150 Rockwell B: 87 | 82.7 | 1/2"x2"x3"  5/8"x6" | blue-yellow, AL 7075 |
| 1035 Cold-Drawn Steel | Brinell: 170 Rockwell B: 86 | 84.8 |  5/8"x6" | red-orange, ST 1035 |
| 1045 Cold-Drawn Steel | Brinell: 187 Rockwell B: 90 Rockwell C: 10 | 95 | 1"x1"x3"  5/8"x6" | red-yellow, ST 1045 |
| C95400 Aluminum Bronze | Brinell: 195 Rockwell B: 94 | 89.9 | 1/2"x3"x3" | BRZ 95400 |
| 4140 Annealed Steel | Brinell: 197 Rockwell B: 92 Rockwell C: 13 | 95 | 1/2"x3"x3" | ST 4140 |
| 4130 (Chromoly) Normalized Alloy Steel | Brinell: 197 Rockwell B: 92 Rockwell C: 13 | 97.2 | 1/4"x5/8"x6" | ST 4130 |
| Gray Cast Iron | Brinell: 183-234 Rockwell B: 97 Rockwell C: 20 | 40 (minimum) |  3/4"x6" | green |
| Buna-N Square O-Ring Cord Stock | Shore A: 70-75 | --- | 3/4"x3/4"x6" | yellow |
| Low-Density Polyethylene | Rockwell R: 60 Shore D: 42 | 1.6 | 1"x1"x6" | red |
| High-Density Polyethylene | Rockwell R: 65 Shore D: 68 | 4.4 | 1"x1"x6" | black |
| UHMW, reprocessed | Rockwell R: 64 | 4.8 | 1/4"x3"x3" |  |
| Polypropylene | Rockwell R: 92 Shore D: 77 | 4.8 | 1"x1"x6" 1/4"x3"x3" | green |
| PVC Type I | Rockwell R: 110 Shore D:80 | 7.3 | 1"x1"x6" | orange-silver |
| PVC Type I | Rockwell R: 114 Shore D: 89 | 7.5 | 1"x1"x6" | orange-gold |
| Acetal (Delrin) | Shore D: 85 | 9.5 | 1/4"x3"x3" |  |
| Polycarbonate | Rockwell M: 70 Rockwell R: 118 | 9.5 | 1/4"x3"x3" |  |
| Cast Acrylic | Rockwell M: 93 | 10 | 1" cube, 1/4"x3"x3" | blue |
| Cast Nylon | Shore D: 80 | 12 | 1/4"x3"x3" |  |