- What is a composite?

  - A composite is anything made using more than one type of material

  - Structure of a composite

    - Usually a continuous phase and a discontinuous phase are present

Composite BeamsIntroduction to Composite Materials

    - Discontinuous phase is usually harder, stronger, and more expensive

      - Called the reinforcing material

    - Continuous phase is usually weaker than discontinuous if used alone

      - Called the matrix of the composite

    - Based on this, who can name a common composite and identify the matrix and the reinforcing material?

      - Most common is steel reinforced concrete where the concrete is the matrix and steel is the reinforcing material

      - Fiberglass

  - Reasons for using composites

    - Allows you to create a material with properties that are superior to those of the individual materials

    - Can exploit the good points of the individual materials and make up for any shortcomings by using the other materials appropriately

- Examples of Composites and Their Use

  - Carbon fiber reinforced polymer wrap on concrete

Examples of Composite Materials Fiber-Wrapped Concrete Column

    - Concrete column alone would fail at 600,000 lb

    - With CFRP wrap, the column withstood 3,000,000 lb which was the capacity of the testing machine

    - Can be used to repair concrete columns that have been damaged

  - UMR Smart Composite Bridge

Examples of Composite MaterialsUMR Smart Composite Bridge

    - Used bonded tubes in construction

      - Built up layer-by-layer

- Lab Procedure

  - We will be working with composite beams made from aluminum and stainless steel sections bolted together

    - Make sure you set up the beam with the stainless steel side up

  - You will need to take several measurements and record them on your data sheet

    - Draw on board

  - There are two strain gages attached to the composite beam

    - One is on the top surface and the other is on the bottom surface

    - Both measure axial strain

  - Place the weight hanger on the beam if using the lathe bed

    - You can select any arbitrary location to place the load

  - Connect the strain gages to the indicators and follow the steps from our previous lab

    - Zero the amp

    - Set the gage factor

    - Balance the load

  - Load the beam in 10 lb increments from 0 to 100 lb

    - Record the axial strains at each 10 lb increment of load

- Calculations

  - Start your calculations by converting your strain values to stresses using

    - These formulas are from Hooke’s law for uniaxial loading

  - Next, use Excel to plot normal stress vs. load for both strain gages on the same graph

    - From linear regression find the slopes of the two lines

  - Use your slopes to find the experimental location of the neutral axis

    - Convert your slopes back into strains per unit load

    - Use the following formula to find the location of the experimental neutral axis

      - This follows from the fact that strain increases linearly with distance from the neutral axis

        - Allows the use of similar triangles

- Composite Beam Theory

  - We will use composite beam theory to calculate theoretical values to compare the experimental values against

    - Can’t directly apply beam theory to a composite beam

      - What assumption of beam theory do we violate if we apply it directly to a composite beam?

      - Due to the fact that the beam is made of more than one material

      - Beam isn’t homogeneous

    - We get around this by transforming the beam cross section into an equivalent cross section made from one material

      - Creates a homogeneous beam that allows use of the usual beam formulas

  - Cross Section Transformation

    - Transform the cross section to being made of only one of the two materials

    - Easiest if you convert the material with the higher E into an equivalent amount of the other material

      - Find the ratio of the modulus of elasticities of the two materials

      - Multiply the width of the stainless steel by n to find the transformed width

      - Creates an equivalent cross section composed entirely of aluminum

      - The height of the stainless steel section does NOT change

        - This ensures the strain and radius of curvature are continuous from one section to the other

  - Theoretical Neutral Axis Location

    - Solve for this using the transformed cross section and the usual method of finding the centroid by applying

      - Make sure all the dimensions you use in this equation come from the transformed cross section

      - The location found for the transformed cross section is the same location of the NA as for the actual composite beam

  - Moment of Inertia About Neutral Axis

    - This should also be calculated using the transformed cross section dimensions

  - Theoretical Stresses per unit Load

    - These are found by looking at a FBD of the beam at the section where the strain gages are located

RA=L2PL

Lg

      - ΣMg=MB−RALg=0

      - MB=RALg=PLgL2L

- Lab Report

  - The report for this lab should be a memo worth 100 points

  - Make sure you attach your initialed data sheet and sample hand calculations

  - Experimental Results

    - Include a table showing the original data collected in lab

    - Include the graph created in Excel and show the linear regression lines along with their equations

    - Calculate the following experimental values

      - Location of neutral axis ()

      - Stress per unit load for each material

    - Calculate the following theoretical values

      - Location of neutral axis

      - Stress per unit load for each material

    - Create a table with the following information

| Experimental |  |  |  |
| --- | --- | --- | --- |
| Theoretical |  |  |  |
| % Difference |  |  |  |

  - Discussion of Results

    - Compare your experimental and theoretical values using % error

    - Discuss how well the composite beam theory predicted your experimental values

    - Describe any reasons for why your results don’t exactly match the theory

    - Research and discuss two different composites

      - Tell what materials are used to create the composite

      - Describe how the materials are joined together and the benefits gained by using them as a composite

- Presentation

  - Each group will write their experimental values for, , and  on the board.