- Beam Deflection

  - Importance

    - As we’ll talk about later in the semester one of the types of engineering failures is excessive elastic deformation

      - So the stresses in the material do not have to reach the yield point for a material to fail

    - We would like to be able predict the amount of deflection for a given loading situation

      - This is where understanding beam deflection becomes a useful tool

  - Assumptions

    - Linear elastic material

      - Same as before

      - We haven’t yielded the material and there is a linear relationship between stress and strain

    - Homogeneous, isotropic material

      - Same throughout

      - Properties the same in all directions

    - Small deformations

      - Allows use of the small angle approximation

    - Pure bending

      - Neglect the shear stresses that are almost always going to be present

      - If the length of the beam is at least 10 times the thickness of the beam then this results in at worst 3% error

- Beam Tables

  - Apply the assumptions of beam deflection theory to common beam loading situations

  - Easy to use

    - Find your given loading situation and read from the table the equation for deflection at a given point on the beam

- Lab Procedure

  - Each group will perform beam deflection tests on two beams

  - One beam is a cantilevered wood beam

  - Other beam will be a simply supported aluminum beam

  - We will use dial indicators to measure the deflection of each beam at two different points

  - Cantilevered Wood Beam

    - Take all measurements required on your data sheet

    - Use a length of roughly 36 inches

    - Set one indicator approximately ½ of the beam length from the cantilevered support

    - Place the other indicator near the end of the beam

    - Zero the indicators with the weight hanger on the beam

    - Apply load in 1 lb increments from 0 to 10 lbs

  - Simply Supported Aluminum Beam

    - Take all measurements required on the data sheet

    - Place the weight hanger on the beam exactly half way between the supports

    - Set one indicator about ¼ of the beam length from the support

    - Set the other indicator about ½ of the beam length from the support

    - Zero the indicators with the weight hanger on the beam

    - Apply load in 5 lb increments from 0 to 50 lbs

- Calculations

  - Start your calculations for both beams by entering your data in Excel

    - Create one graph for each beam

      - Plot deflection vs. load for the two indicators

      - Use linear regression to find the slope yiPexp of the regression line through the points

  - Beam Theory

    - We will use beam deflection theory to evaluate our experimental results

      - We will compare our deflection per unit load values found for the aluminum beam to the theoretical values

      - We will use the beam theory to calculate the modulus of elasticity of the wood beam using our experimental deflection per unit load values

    - Aluminum beam

      - Calculate theoretical values for yiPusing the following formula from the beam table

        - yiPth=x3L2−4x248EalINA

      - Use Eal=10.6×106psi

      - Compare the deflection per unit load value from beam theory to the experimental value using percent difference

        - Will have two comparisons to make

          - One for each indicator

    - Wood Beam

      - The modulus of elasticity of wood is usually not very well known so we will solve for it

      - Calculate the experimental value for the modulus of elasticity of the beam using

        - Ewood=x23L−x6INAyiPexp

      - Compare your experimental E to the appropriate reference value on the data sheet

        - Again you will have two % difference comparisons to make

- Lab Report

  - The report for this lab should be a memo written by your group worth 100 points

  - Include the original, initialed data sheet and a set of hand calculations

  - Experimental Results

    - Include a table showing your original data

    - Show the graphs created in Excel for linear regression

      - Make sure you show the regression lines and their equations on the graphs

    - Calculate the theoretical value of deflection per unit load for the aluminum beam

    - Calculate the experimental modulus of elasticity for wood

    - Create a table summarizing your experimental and theoretical values

  - Discussion of Results

    - Compare your experimental and theoretical or reference values using percent errors

    - Give reasons for any major differences

    - Explain whether the assumptions of the beam deflection theory were well met or not

    - Compare your results for the aluminum and wood beams and tell which material worked better for the beam theory

- Presentation

  - Each group will come to the board and write your experimental values of yiPexpfor the aluminum and Ewood from the wood beam test

  - Two groups will be randomly selected to answer questions about the lab