Spring Testing

Lab Procedure

The purpose of this first assignment is to give everyone a chance to work with a spreadsheet program such as Excel.  We will plot data points and perform linear regression on the data.  You will be given sample data sets from tensile tests on steel and aluminum and you will collect load versus deflection data for a spring.  For the sample tensile test data sets, follow the steps in the Sample Tensile Test Data Sets section.  For your spring data, follow the steps in the Experimental Spring Data section.

Sample Tensile Test Data Sets:

Each person will be assigned two sample data sets to analyze.

| Assigned data sets: |  |
| --- | --- |

The sample data sets can be found at:

Course WebsiteLessonsIntroduction, Spring TestingTension Test Data

After obtaining your sample data sets, follow the steps listed below:

- In Excel set up tables for your data sets.  Include appropriate column labels for your data.  Input your data sets with separate columns for the load and deformation.  Make sure you label the data sets so I know which ones you used.

- Record the gage length and specimen diameter at a convenient location in your spreadsheet.  The specimens have a circular cross-section.  Use this information to calculate and record the cross-sectional area of the specimen.

- Add appropriately labeled columns and generate calculated values of stress and strain.  Stress can be calculated using Eq. (1), and strain can be calculated using Eq. (2).  All equations are shown in the equations section on the next page.

- Perform linear regression on your calculated stress vs. strain values.  Use all the data points for each set in your regression analysis.  Remember to use the stresses as y-values and the strains as x-values.  Your regression equation will have the form y = mx+b where y is the stress and x is the strain.

- Create a graph of your stress vs. strain data.  Include your sample data points and the linear regression line along with its equation.  You need to create a separate graph for each data set.  Start the scales at zero and extend them slightly beyond the limits of your data.  Your graph needs to have a title, and the axes should be labeled.

If you need a refresher on spreadsheets there are help files available at:

Course WebsiteLessonsIntroduction, Spring TestingSpreadsheets

You can also stop by my office and I will help you with the assignment.

Experimental Spring Data:

After performing the test on the assigned spring you should have data points for deflection at different loads.  You will need to complete the following for your load vs. deflection data.

- Record the spring # for your tested spring at a convenient location in your spreadsheet.

- Set up a table for your data set.  Input your data with separate columns for the load and deflection.

- Perform linear regression on your experimental load vs. deflection data.  Use all the data points in your regression analysis.  Remember to use the loads as y-values and the deflections as x-values.

- The slope found from your regression analysis is by definition the spring constant for your spring as shown in Eq. (3).

- Create a graph of your load vs. deflection data.  Include your data points and the linear regression line along with its equation.

Lab Report

This week’s lab report is an individual assignment worth 50 points.  The report format for this lab is a memo.  Answer the following questions in the appropriate section of your memo.

- Do the sample tensile test data sets appear to show a linear relationship between stress and strain?

- What was the spring constant for your tested spring?  How would knowing the spring constant be useful in a real-world application?

- Did any of the data points seem out of place for either the sample data or the spring data?

- Were there any parts of the assignment that were difficult for you?

Also, include or attach the following:

- A table summarizing the slope and y-intercept for each sample tension test data set.

- The graphs of your stress vs. strain data.

- The graph of load vs. deflection for your spring data

Remember to follow the guidelines for graphs and tables that I gave you.  Make sure you mention in your memo where your tables and figures are so I know where to find them.  Be sure to complete and attach a set of hand calculations showing your calculations of stress and strain.  You only need to perform hand calculations for one data point in one of your sample data sets.  There is no data sheet for this lab so you do not need to worry about attaching one.

Presentation

At the beginning of the next lab I will ask each of you to write your calculated spring constant on the board.  This is all that will be required for the presentation.

Equations

(1) Stress					(2) Strain			(3) Spring Constant

k=ΔFΔL

σ = stress, psi				= strain, in/in		k= spring constant, lb/in

F= load, lb				= deformation, in		ΔF= change in force, lb

A= cross-sectional area, in2		L0= gage length, in		ΔL= deflection, in