- What is strain transformation and how is it useful?

  - This lab will help you better understand the concepts of stress and strain transformation you have learned about in IDE 110

  - Allows you to determine the principal strains on an element and the orientation of the element with the principal strains

  - Will help you understand what is happening to the strain gages we have used in previous labs

- Lab Procedure

  - This lab involves some of the more difficult calculations we will do this semester

    - Don’t be overwhelmed, together we’ll make it through it

    - Ask for help if you need it

  - Hand out Transparencies and Data Sheets

    - Draw an example square on the board

θ

    - Before testing, measure using calipers or protractor

      - Probably best if the same person makes the same measurements before and after testing

      - Lengths of the sides (4 per square)

        - Use the points of the calipers for your measurements

        - Be consistent when measuring- either always outside lines or always inside lines

        - Accurate measurements will give better results

      - Angles between sides (4 per square)

        - Your measurements will be in degrees

        - It is okay to only use degrees on your data sheet

        - For calculations be absolutely sure you work in radians

      - Orientation angle for each square (1 per square)

        - Assume the top square is perfectly aligned horizontally and vertically

        - Align top square with 90º or 270º line and then find the orientation angle

    - Do this for all four squares and record in the left column of squares on your data sheet

  - Take your transparency sheet to a UTM

    - Load the sheet in the grips

    - The sheet will be stretched about 2.5 inches

    - The sheet will be permanently deformed from its original dimensions

    - Be careful not to touch the ink on the sheet after testing since it will come off and ruin your results

  - After testing

    - Make the same measurements as before except you don’t need to measure orientation angle

      - Lengths of the sides (4 per square)

      - Angles between sides (4 per square)

    - Record your final measurements in the middle column on your data sheet

- Calculations

  - Normal Strain

    - Same as what we used in the tension testing lab

    - Elongation of a line segment per unit length

    - ε=ΔLL

    - For the squares

      - Two sides are aligned with the x and y directions for each square

      - Find the average normal strains for the x and y directions

      - Example

      - Record calculated values in the right column of your data sheet

  - Shear Strain

    - Defined as change in the angle between two line segments that were originally perpendicular

    - We will assume for our calculations that all the squares have 90º corners before testing

      - You still need to measure the initial angles

        - May help explain why your results aren’t perfect

    - For the squares

      - Calculate the average shear strain for each square

      - Example

- Principal Strains

  - The normal and shear strains we just found were the normal and shear strains in a coordinate system aligned with the arbitrary x-y coordinate  systems of the squares on the transparency

  - Really want to find the maximum normal and shear strains in the transparency sheet

  - All squares should experience the same principal normal and shear strains since the entire transparency sheet is subjected to the same strain

    - St. Venant’s principal from Photoelasticity

  - The principal strains can be calculated using either the given formulas or Mohr’s circle

    - You can use either method to find your results

    - My recommendation would be to use Mohr’s circle for the one square you perform hand calculations on and then program the equations in Excel

      - This gives you a very good way to double check that everything has been correctly entered in Excel

    - The formulas from Mohr’s circle are the same as if you just use formulas, however Mohr’s circle gives a good visual check of your calculations

  - Mohr’s Circle

    - Mohr’s circle represents all the possible combinations of normal and shear strain on an element as the element is rotated

      - A 1º rotation of the strain element corresponds to a 2º rotation around Mohr’s circle in the same direction

ε2

X

ε1

εavg

Y

ε

R

2θp

  - Mohr’s Circle Equations

R

    - εavg=εx+εy2

    - εdif=εx−εy2

    - R=εdif2+γxy22

    - ε1,2=εavg±R

    - γmax=2R

    - θp=12tan−1γxy2εdif

  - Example

- Assignment

  - The report for this lab should be a memo worth 100 points

  - Need to complete the calculations discussed and report the results in typed tables

  - Need to show hand calculations for one square

  - Also remember to include your original data sheet

    - Create a table that includes the following information

| Square # | Θ | εx | εy | γxy | θp | ε1 | ε2 | γmax |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

    - Also make tables that compare the following using % differences

      - % Difference formula for experimental values

      - θ to θp for each square using % difference

        - θ to θp should be the same if the load is applied axially and your measurements are accurate

| Square # | θ | θp | % Diff. |
| --- | --- | --- | --- |

      - Compare ε1 and ε2 values for all 4 squares

          - The ε1 values should be the same for all four squares as should the ε2 values

          - For your % differences report the largest % difference between squares along with the smallest

| Square # | ε1 | ε2 |
| --- | --- | --- |
| Largest % Diff. |  |  |
| Smallest % Diff. |  |  |

      - Compare  and from square 1 to  and for square 1

        - Since square 1 is aligned along the axial load, and for square 1 should be the principal normal strains

      - Compare γmax for all 4 squares

          - Again in theory they should all be the same

          - Report the largest and smallest % differences among the four values

| Square # | γmax |
| --- | --- |
| Largest % Diff. |  |
| Smallest % Diff. |  |

      - Compare your for square 4 to

          - Since square 4 is 45º from the axis of loading, it has the maximum shear strains

- Presentation

  - Each group will come to the board and fill in the following information for square 1 and square 4 from your data set

| Square # |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 4 |  |  |  |  |

  - Two random groups will then be asked questions about the lab.