- What is thermal expansion?

  - Thermal expansion is when the dimensions of an object change due to a temperature change of the object

  - What is temperature?

    - Temperature is a measure of the total kinetic energy of the atoms in a system

    - Solid

      - Measures the vibrational energy of the atoms

      - Does not measure the kinetic energy of the overall object

  - Thermal expansion examples

ThermalBridges

Thermal Power Lines

  - Thermal shock

    - Extreme case of thermal expansion

    - If the temperature changes very quickly and the material is brittle

    - Can lead to cracking of the material

    - Who can give an example of thermal shock?

      - Example is ice in a glass of water

- Coefficient of Thermal Expansion

  - Usually abbreviated as CTE

  - Measures the amount of change in a dimension as the temperature changes

  - CTE is inversely proportional to melting point

    - Low melting point materials have high CTE

    - Due to strength of intermolecular bonds in material

      - Weaker in low MP material than in high MP material

  - Formula

    - α=Lf−L0L0Tf−T0=ΔLL0ΔT

- Lab Procedure

  - Each of you will work with your individual objects and go through the following steps

  - Determine the material your object is made from

    - May have to make a guess

    - Look at manufacturer websites to get a better idea

    - Library also has several good references on materials

    - A good website is www.matweb.com

    - If you have trouble, just ask me and I will help you determine the material

  - Measure and record at least two dimensions on your object using calipers

    - Dimensions should be of different types

    - Can use marker to mark a reference location

    - If your material is shiny you will need to darken an area for the infrared thermometer to get an accurate reading

  - Measure the temperature of your object

    - Make measurement before placing in freezer

    - Use the infrared thermometer to measure close to your dimension measurement location

    - May want to take a readings at a couple of different locations and average

  - Remember to have your data sheet initialed by TA or me before leaving

  - Wait at least a day to come back and take final measurements

    - Can come in any time the lab is open

    - Measure temperature before using calipers to measure dimensions

      - Measure the dimensions at the same location as before

    - Measure the temperature again

      - Handling the item may have warmed it up

      - Use the average of your temperature measurements before and after using the calipers as the final temperature

    - You do not need to get any initials for your final measurements

- Assignment

  - Formal Report

    - Worth 100 points

    - Describe how your group selected the objects to test

    - Describe how you determined what materials your objects are made of

    - Include a table showing your experimental data

    - Look up reference values of CTE and melting point for your materials along with 8 other reference properties of your materials (10 total reference properties for each material)

      - Compare your experimental values for CTE to the reference values using % error

    - Discuss whether your materials follow the general guideline that a higher melting point leads to a lower CTE

    - Be sure to show a sample calculation of CTE

    - Attach your initialed original data sheets

- Presentation

  - List your materials, experimental CTE, and reference melting point on the board

- What is photoelasticity?

  - Photoelasticity is a technique used in stress analysis

  - When a photoelastic material has a load applied and is viewed under polarized light

    - Colorful patterns are seen that are directly proportional to the stresses and strains in the material

    - Every 3 colors represents an order of magnitude change in the stress and strain

  - Can be used on actual parts by applying a photoelastic coating

PhotoelasticityIntroduction

PhotoelasticityApplications